FINAL REPORT

GPS Analysis and Network adjustment of the 1999 Co-Operative GPS Survey of the

County of San Diego

### Matthijs van Domselaar

California Spatial Reference Center

Scripps Institution of Oceanography

#### University of California, San Diego

9500 Gilman Drive MC 0225

La Jolla, CA 92093-0225

July 27, 2000

# Introduction

On December 3, 1999,  a cooperative GPS densification effort was performed in San Diego county to improve the accuracy of the regional control network. The California Spatial Reference Center (CSRC) was contracted to perform the data reduction and network adjustment.

The survey was conducted with a wide variety of equipment, donated by various organizations. The network was observed in two sessions of three hours, from 17:00 - 20:00 UTC and from 20:30 - 23:30 UTC.

# Analysis and Coordinate Adjustment

The data were reduced using version 9.93 of the GAMIT [King and Bock, 2000] package, and version 5.0 of the GLOBK [Herring, 2000] package . The GAMIT software is a high precision geodetic software package that was used to perform a weighted least squared adjustment using the dual-frequency phase and range observations. Ambiguities were fixed. Zenith neutral delays and tropospheric gradients were estimated and the data were dynamically weighted using an elevation dependent weighting scheme based on the actual quality of the phase data.

We used a three step procedure to adjust the network. First, the global data were analyzed to obtain high precision GPS orbits. These were the same orbits that were submitted to the International GPS Service (IGS) as part of the routine operations of the Scripps Orbit and Permanent Array Center (SOPAC). SOPAC is one of the 7 official IGS analysis centers that contribute to the IGS precise ephemerides.

Secondly, all data from 150+ Continuously Operated Reference Stations (CORS) in California were analyzed using these orbits, including the National CORS stations BILL, BLYT, BRIB, CMBB, DHLG, PBL2, S300 and VAN1. The data from the San Diego GPS  (SDPGS) Survey were analyzed in the same manner as the CORS stations. Selected stations  (MONP, MVFD, PMOB, TRAK and SIO3) from the Southern California Integrated GPS Network were included in the SDGPS network analysis.

The three sets of solution files (global, regional CORS, SDGPS) were then used as input to the GLOBK software, which is a high precision network adjustment tool, based on a Kalman filter technique [Herring et al., 1990]. Each of these analysis files contain the complete geodetic solution, including the full variance-covariance matrix of the station parameters and the corresponding adjustments. The GLOBK package is especially powerful in combining individual sessions (different networks, different days) of observations to obtain an estimate of station coordinates averaged over a multi-day experiment. The final coordinates and statistics presented in this report are derived from the GLOBK adjustment.

The underlying reference frame is based on a subset of coordinates and velocities of the National Spatial Reference System (NSRS) as provided by the National Geodetic Survey (Dale Pursell, personal communication). ITRF97 [ Boucher et al., 1999] versions of these data were provided for the following stations: BILL, BLYT, DHLG, GOL2, CMDD, CNDR, S300 and VAN1 (See Table 1). These values were used to impose the reference frame by performing a seven parameter transformation after all of the GAMIT solutions are combined and loosely constrained. This rigorous implementation of the NSRS ensures an optimal definition the the SDGPS coordinates in this reference system. To obtain NAD83 coordinates, the Horizontal Time Dependent Positioning Tool (HTDP), version 2.3, developed by NGS was used to transform the ITRF97 coordinates into the NAD83 system [Craymer et al., 1999] for reference epochs 1991.35 and 1999.92.

# Results

Since the SDGPS stations were only observed for two sessions, the regular statistical methods (e.g, coordinate repeatabilities) to quantify the precision of the coordinates could not be applied. However, the difference between the station-coordinates of the two sessons is still relevant and provided in Table 2. The mean difference was 3/3/10 mm in North, East and Vertical direction, respectively.

The two loosely constrained solutions were then combined with global data and regional data, after which the NSRS was imposed. The final NAD83 coordinates are given in Appendix A (epoch 1991.35) and Appendix B (epoch 1999.92).

Table 1: ITRF97 coordinates and velocities that define the regional realization of the NSRS, provided by NGS. Coordinates are with respect to the Monument Reference Point.

Table 2: Coordinate difference between the two observed sessions in North, East and Vertical direction in meters.

## References

Boucher, C, Z. Altamimi and P. Sillard, Results and Analysis of the ITRF97, IERS 	Technical Note 26, Observatoire de Paris, 1999.

Craymer, M.,  R. Ferland, R. Snay, Realization and Unification of NAD83 in Canada and

the U.S. via the ITRF. Proceedings of the International Symposium of the

International Association of Geodesy, Section II, "Towards an Integrated

Geodetic Observing System (IGGOS)", Munich, October 5-9, 1998. Revised, 11

March 1999.

Herring, T.A., J.L. Davis, and I.I. Shapiro, Geodesy by radio astronomy: the 	application of Kalman filtering to very long baseline interferometry, 		J. Geophys. Res., 95, 12,561 - 12,581, 1990.

Herring, T.A., Documentation of the GLOBK Software, Massachussets Intitute of 	Technology, 2000.

King, R.W. and Bock, Y., Documentation of the GAMIT GPS Analysis Software, 	Massachussets Intitute of Technology and Scripps Institution of 	Oceanography, 2000.

Appendix A : SDGPS coordinates in NAD83 and ITRF97, reference epoch 05-07-1991

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

0100_GPS

LATITUDE     32 48 26.85819N   32 48 26.83295N

LONGITUDE   117 07 27.73689W  117 07 27.68378W

ELLIP. HT.            92.008            92.773 m

X               -2446554.841      -2446554.097 m

Y               -4775976.777      -4775978.354 m

Z                3436080.730       3436080.491 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

0379_GPS

LATITUDE     33 17  5.30565N   33 17  5.28065N

LONGITUDE   116 40 30.86082W  116 40 30.80992W

ELLIP. HT.           841.032           841.795 m

X               -2396362.631      -2396361.930 m

Y               -4769770.791      -4769772.330 m

Z                3480868.506       3480868.281 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

0442_GPS

LATITUDE     33 17 59.86068N   33 17 59.83535N

LONGITUDE   117 08 55.30375W  117 08 55.25121W

ELLIP. HT.           222.321           223.072 m

X               -2435037.766      -2435037.038 m

Y               -4748524.939      -4748526.500 m

Z                3481933.908       3481933.667 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

0505_GPS

LATITUDE     33 23 10.61488N   33 23 10.58943N

LONGITUDE   117 10 20.94809W  117 10 20.89580W

ELLIP. HT.           230.331           231.079 m

X               -2434609.052      -2434608.332 m

Y               -4742837.727      -4742839.284 m

Z                3489936.546       3489936.303 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

0CD3_GPS

LATITUDE     32 46  0.07901N   32 46  0.05383N

LONGITUDE   116 58 57.38899W  116 58 57.33602W

ELLIP. HT.           370.126           370.896 m

X               -2435947.551      -2435946.807 m

Y               -4784405.960      -4784407.536 m

Z                3432429.904       3432429.669 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

0DAY_GPS

LATITUDE     32 43 33.98891N   32 43 33.96377N

LONGITUDE   116 52 58.06524W  116 52 58.01235W

ELLIP. HT.           361.727           362.501 m

X               -2428706.786      -2428706.041 m

Y               -4790807.349      -4790808.926 m

Z                3428639.991       3428639.758 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

0P62_GPS

LATITUDE     33 16 12.56587N   33 16 12.54092N

LONGITUDE   116 39 18.26975W  116 39 18.21886W

ELLIP. HT.           901.304           902.068 m

X               -2395106.466      -2395105.765 m

Y               -4771455.773      -4771457.311 m

Z                3479543.007       3479542.783 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

1107_GPS

LATITUDE     33 07 46.19948N   33 07 46.17412N

LONGITUDE   117 16 37.13164W  117 16 37.07853W

ELLIP. HT.            59.779            60.533 m

X               -2450347.515      -2450346.776 m

Y               -4752144.807      -4752146.378 m

Z                3466027.667       3466027.425 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

1108_GPS

LATITUDE     33 14  1.60355N   33 14  1.57856N

LONGITUDE   116 41 36.00858W  116 41 35.95732W

ELLIP. HT.           841.116           841.880 m

X               -2399263.528      -2399262.819 m

Y               -4771787.146      -4771788.690 m

Z                3476135.461       3476135.236 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

A007_GPS

LATITUDE     33 23 55.09270N   33 23 55.06724N

LONGITUDE   117 11 25.27492W  117 11 25.22263W

ELLIP. HT.           434.610           435.358 m

X               -2435821.358      -2435820.638 m

Y               -4741559.065      -4741560.622 m

Z                3491193.131       3491192.888 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

A009_GPS

LATITUDE     33 20  8.25750N   33 20  8.23207N

LONGITUDE   117 14 21.65450W  117 14 21.60187W

ELLIP. HT.           176.193           176.942 m

X               -2441535.617      -2441534.890 m

Y               -4742700.129      -4742701.691 m

Z                3485214.199       3485213.956 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

CC01_GPS

LATITUDE     33 22 35.57553N   33 22 35.55008N

LONGITUDE   117 13 39.99833W  117 13 39.94586W

ELLIP. HT.           223.690           224.438 m

X               -2439454.091      -2439453.368 m

Y               -4741009.300      -4741010.859 m

Z                3489031.436       3489031.192 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

CC02_GPS

LATITUDE     33 26  5.65768N   33 26  5.63218N

LONGITUDE   117 14 38.37162W  117 14 38.31930W

ELLIP. HT.           299.974           300.719 m

X               -2439193.508      -2439192.789 m

Y               -4737207.078      -4737208.635 m

Z                3494476.753       3494476.508 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

CC03_GPS

LATITUDE     33 25 55.33327N   33 25 55.30778N

LONGITUDE   117 13 38.83761W  117 13 38.78533W

ELLIP. HT.           236.843           237.588 m

X               -2437882.193      -2437881.475 m

Y               -4738019.898      -4738021.454 m

Z                3494176.509       3494176.265 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

CC04_GPS

LATITUDE     33 03 53.80729N   33 03 53.78192N

LONGITUDE   117 17 36.35896W  117 17 36.30575W

ELLIP. HT.            19.077            19.831 m

X               -2453489.002      -2453488.261 m

Y               -4754884.947      -4754886.521 m

Z                3460007.677       3460007.433 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

CC05_GPS

LATITUDE     33 03 55.17472N   33 03 55.14937N

LONGITUDE   117 16 44.53121W  117 16 44.47802W

ELLIP. HT.            62.362            63.117 m

X               -2452300.266      -2452299.525 m

Y               -4755513.093      -4755514.667 m

Z                3460066.597       3460066.355 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

CC06_GPS

LATITUDE     32 59 43.34443N   32 59 43.31933N

LONGITUDE   116 51 29.29360W  116 51 29.24107W

ELLIP. HT.           429.163           429.930 m

X               -2419349.551      -2419348.815 m

Y               -4777446.550      -4777448.116 m

Z                3453762.014       3453761.783 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

CC08_GPS

LATITUDE     33 03  5.15507N   33 03  5.13022N

LONGITUDE   116 34 49.02599W  116 34 48.97430W

ELLIP. HT.          1473.023          1473.797 m

X               -2395028.602      -2395027.879 m

Y               -4786874.941      -4786876.494 m

Z                3459544.446       3459544.226 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

CC09_GPS

LATITUDE     33 01 28.18744N   33 01 28.16261N

LONGITUDE   116 33 12.81931W  116 33 12.76753W

ELLIP. HT.          1474.808          1475.583 m

X               -2393524.532      -2393523.806 m

Y               -4789450.047      -4789451.602 m

Z                3457040.573       3457040.354 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

CC10_GPS

LATITUDE     33 13 22.41283N   33 13 22.38735N

LONGITUDE   117 20 39.64624W  117 20 39.59310W

ELLIP. HT.           -19.942           -19.192 m

X               -2453298.931      -2453298.195 m

Y               -4744166.561      -4744168.132 m

Z                3474653.586       3474653.340 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

CC11_GPS

LATITUDE     33 14 42.82943N   33 14 42.80396N

LONGITUDE   117 19 22.52610W  117 19 22.47303W

ELLIP. HT.           -15.225           -14.476 m

X               -2450903.542      -2450902.806 m

Y               -4743880.680      -4743882.249 m

Z                3476728.418       3476728.172 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

CC12_GPS

LATITUDE     33 24 26.57693N   33 24 26.55145N

LONGITUDE   117 16 27.45360W  117 16 27.40112W

ELLIP. HT.           275.803           276.549 m

X               -2442459.668      -2442458.946 m

Y               -4737392.994      -4737394.553 m

Z                3491915.516       3491915.271 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

CC13_GPS

LATITUDE     33 20 26.98967N   33 20 26.96430N

LONGITUDE   117 06 37.53024W  117 06 37.47794W

ELLIP. HT.           200.682           201.432 m

X               -2430722.499      -2430721.777 m

Y               -4747917.792      -4747919.349 m

Z                3485709.813       3485709.573 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

CC14_GPS

LATITUDE     33 13 25.40420N   33 13 25.37897N

LONGITUDE   117 06 38.26983W  117 06 38.21715W

ELLIP. HT.           404.540           405.294 m

X               -2434065.417      -2434064.684 m

Y               -4754405.469      -4754407.031 m

Z                3474963.257       3474963.021 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

CC15_GPS

LATITUDE     32 43 25.76138N   32 43 25.73619N

LONGITUDE   116 58  3.20236W  116 58  3.14939W

ELLIP. HT.           121.182           121.953 m

X               -2435761.758      -2435761.013 m

Y               -4787150.973      -4787152.551 m

Z                3428296.719       3428296.483 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

CC16_GPS

LATITUDE     32 49 41.40742N   32 49 41.38217N

LONGITUDE   117 09 30.15234W  117 09 30.09920W

ELLIP. HT.            99.287           100.051 m

X               -2448823.544      -2448822.800 m

Y               -4773421.928      -4773423.505 m

Z                3438014.725       3438014.486 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

CC18_GPS

LATITUDE     32 55  9.47961N   32 55  9.45448N

LONGITUDE   116 58 50.86787W  116 58 50.81501W

ELLIP. HT.           254.719           255.485 m

X               -2431587.658      -2431586.917 m

Y               -4776216.218      -4776217.789 m

Z                3446587.585       3446587.351 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

CC19_GPS

LATITUDE     33 05 43.56287N   33 05 43.53795N

LONGITUDE   116 38 46.16091W  116 38 46.10924W

ELLIP. HT.          1080.727          1081.497 m

X               -2399188.279      -2399187.558 m

Y               -4781443.715      -4781445.268 m

Z                3463420.568       3463420.345 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

CC20_GPS

LATITUDE     33 03 55.11909N   33 03 55.09416N

LONGITUDE   116 39  7.96285W  116 39  7.91103W

ELLIP. HT.          1157.974          1158.745 m

X               -2400540.804      -2400540.080 m

Y               -4782877.965      -4782879.520 m

Z                3460662.921       3460662.698 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

HIGH_GPS

LATITUDE     33 01 17.14908N   33 01 17.12397N

LONGITUDE   116 54 41.33763W  116 54 41.28504W

ELLIP. HT.           424.182           424.947 m

X               -2423082.098      -2423081.362 m

Y               -4773784.328      -4773785.893 m

Z                3456182.843       3456182.611 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

HOSK_GPS

LATITUDE     33 04 29.67753N   33 04 29.65263N

LONGITUDE   116 37 32.98381W  116 37 32.93210W

ELLIP. HT.          1215.911          1216.683 m

X               -2398099.526      -2398098.804 m

Y               -4783506.811      -4783508.364 m

Z                3461586.880       3461586.658 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

S301_GPS

LATITUDE     33 22 31.09698N   33 22 31.07129N

LONGITUDE   117 33 54.58990W  117 33 54.53661W

ELLIP. HT.            -5.936            -5.194 m

X               -2467275.488      -2467274.756 m

Y               -4726459.748      -4726461.322 m

Z                3488789.887       3488789.634 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

S302_GPS

LATITUDE     33 21 19.55175N   33 21 19.52631N

LONGITUDE   117 14 56.80794W  117 14 56.75535W

ELLIP. HT.           184.447           185.195 m

X               -2441794.209      -2441793.483 m

Y               -4741216.671      -4741218.232 m

Z                3487053.655       3487053.411 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

S317_GPS

LATITUDE     32 49  6.35134N   32 49  6.32608N

LONGITUDE   117 08 52.23407W  117 08 52.18094W

ELLIP. HT.            90.205            90.969 m

X               -2448209.614      -2448208.869 m

Y               -4774386.127      -4774387.704 m

Z                3437102.269       3437102.030 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

S318_GPS

LATITUDE     32 54 42.60076N   32 54 42.57564N

LONGITUDE   116 55 52.81886W  116 55 52.76608W

ELLIP. HT.           209.071           209.839 m

X               -2427650.342      -2427649.602 m

Y               -4778680.411      -4778681.981 m

Z                3445867.638       3445867.405 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

S436_GPS

LATITUDE     32 50 39.84378N   32 50 39.81871N

LONGITUDE   116 48  7.48049W  116 48  7.42781W

ELLIP. HT.           390.398           391.170 m

X               -2418762.661      -2418761.920 m

Y               -4787904.730      -4787906.301 m

Z                3439685.197       3439684.967 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

SD03_GPS

LATITUDE     33 19 54.33622N   33 19 54.31083N

LONGITUDE   117 09 31.72522W  117 09 31.67277W

ELLIP. HT.            60.096            60.846 m

X               -2434930.095      -2434929.370 m

Y               -4746250.705      -4746252.264 m

Z                3484792.062       3484791.821 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

SD07_GPS

LATITUDE     33 23 15.88137N   33 23 15.85640N

LONGITUDE   116 38 14.04619W  116 38 13.99614W

ELLIP. HT.          1420.794          1421.554 m

X               -2390602.038      -2390601.356 m

Y               -4766182.961      -4766184.486 m

Z                3490727.143       3490726.919 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

SD08_GPS

LATITUDE     33 21 50.51393N   33 21 50.48852N

LONGITUDE   117 08 37.74476W  117 08 37.69246W

ELLIP. HT.           329.508           330.257 m

X               -2432892.874      -2432892.153 m

Y               -4745337.093      -4745338.650 m

Z                3487930.186       3487929.944 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

SD15_GPS

LATITUDE     33 04 27.64026N   33 04 27.61540N

LONGITUDE   116 35 52.57465W  116 35 52.52302W

ELLIP. HT.          1249.128          1249.900 m

X               -2395798.444      -2395797.723 m

Y               -4784729.154      -4784730.706 m

Z                3461552.404       3461552.184 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

SD21_GPS

LATITUDE     32 49 26.15383N   32 49 26.12891N

LONGITUDE   116 37  9.25249W  116 37  9.19997W

ELLIP. HT.          1064.373          1065.151 m

X               -2404276.609      -2404275.867 m

Y               -4797205.844      -4797207.413 m

Z                3438142.985       3438142.762 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

SD33_GPS

LATITUDE     32 40 13.54418N   32 40 13.51905N

LONGITUDE   116 49 42.97429W  116 49 42.92144W

ELLIP. HT.           188.394           189.171 m

X               -2425614.132      -2425613.387 m

Y               -4795948.955      -4795950.532 m

Z                3423349.900       3423349.668 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

T228_GPS

LATITUDE     33 18 26.62418N   33 18 26.59879N

LONGITUDE   117 12 29.72611W  117 12 29.67346W

ELLIP. HT.            26.097            26.847 m

X               -2439690.783      -2439690.054 m

Y               -4745442.533      -4745444.096 m

Z                3482515.305       3482515.063 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

VITA_GPS

LATITUDE     33 21 51.62707N   33 21 51.60165N

LONGITUDE   117 09 44.29659W  117 09 44.24424W

ELLIP. HT.           130.030           130.778 m

X               -2434339.175      -2434338.454 m

Y               -4744386.857      -4744388.415 m

Z                3487849.124       3487848.882 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 05-07-1991)

INPUT COORDINATES   OUTPUT COORDINATES

YUNG_GPS

LATITUDE     33 25 48.77827N   33 25 48.75276N

LONGITUDE   117 08 40.79254W  117 08 40.74048W

ELLIP. HT.           318.630           319.377 m

X               -2431115.291      -2431114.576 m

Y               -4741697.351      -4741698.904 m

Z                3494053.022       3494052.778 m

Appendix B: SDGPS coordinates in NAD83 and ITRF97, reference epoch 12-03-1999

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

0100_GPS

LATITUDE     32 48 26.85819N   32 48 26.84203N

LONGITUDE   117 07 27.73689W  117 07 27.69297W

ELLIP. HT.            92.008            92.779 m

X               -2446554.841      -2446554.242 m

Y               -4775976.777      -4775978.115 m

Z                3436080.730       3436080.729 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

0379_GPS

LATITUDE     33 17  5.30565N   33 17  5.28918N

LONGITUDE   116 40 30.86082W  116 40 30.81704W

ELLIP. HT.           841.032           841.805 m

X               -2396362.631      -2396362.034 m

Y               -4769770.791      -4769772.126 m

Z                3480868.506       3480868.506 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

0442_GPS

LATITUDE     33 17 59.86068N   33 17 59.84431N

LONGITUDE   117 08 55.30375W  117 08 55.25966W

ELLIP. HT.           222.321           223.082 m

X               -2435037.766      -2435037.167 m

Y               -4748524.939      -4748526.272 m

Z                3481933.908       3481933.904 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

0505_GPS

LATITUDE     33 23 10.61488N   33 23 10.59847N

LONGITUDE   117 10 20.94809W  117 10 20.90396W

ELLIP. HT.           230.331           231.090 m

X               -2434609.052      -2434608.453 m

Y               -4742837.727      -4742839.059 m

Z                3489936.546       3489936.541 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

0CD3_GPS

LATITUDE     32 46  0.07901N   32 46  0.06283N

LONGITUDE   116 58 57.38899W  116 58 57.34517W

ELLIP. HT.           370.126           370.901 m

X               -2435947.551      -2435946.953 m

Y               -4784405.960      -4784407.299 m

Z                3432429.904       3432429.905 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

0DAY_GPS

LATITUDE     32 43 33.98891N   32 43 33.97273N

LONGITUDE   116 52 58.06524W  116 52 58.02150W

ELLIP. HT.           361.727           362.506 m

X               -2428706.786      -2428706.188 m

Y               -4790807.349      -4790808.689 m

Z                3428639.991       3428639.993 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

0P62_GPS

LATITUDE     33 16 12.56587N   33 16 12.54940N

LONGITUDE   116 39 18.26975W  116 39 18.22599W

ELLIP. HT.           901.304           902.077 m

X               -2395106.466      -2395105.869 m

Y               -4771455.773      -4771457.108 m

Z                3479543.007       3479543.007 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

1107_GPS

LATITUDE     33 07 46.19948N   33 07 46.18321N

LONGITUDE   117 16 37.13164W  117 16 37.08753W

ELLIP. HT.            59.779            60.540 m

X               -2450347.515      -2450346.916 m

Y               -4752144.807      -4752146.140 m

Z                3466027.667       3466027.663 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

1108_GPS

LATITUDE     33 14  1.60355N   33 14  1.58711N

LONGITUDE   116 41 36.00858W  116 41 35.96480W

ELLIP. HT.           841.116           841.890 m

X               -2399263.528      -2399262.931 m

Y               -4771787.146      -4771788.481 m

Z                3476135.461       3476135.461 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

A007_GPS

LATITUDE     33 23 55.09270N   33 23 55.07630N

LONGITUDE   117 11 25.27492W  117 11 25.23079W

ELLIP. HT.           434.610           435.368 m

X               -2435821.358      -2435820.760 m

Y               -4741559.065      -4741560.397 m

Z                3491193.131       3491193.126 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

A009_GPS

LATITUDE     33 20  8.25750N   33 20  8.24113N

LONGITUDE   117 14 21.65450W  117 14 21.61035W

ELLIP. HT.           176.193           176.951 m

X               -2441535.617      -2441535.019 m

Y               -4742700.129      -4742701.461 m

Z                3485214.199       3485214.194 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

CC01_GPS

LATITUDE     33 22 35.57553N   33 22 35.55915N

LONGITUDE   117 13 39.99833W  117 13 39.95417W

ELLIP. HT.           223.690           224.448 m

X               -2439454.091      -2439453.492 m

Y               -4741009.300      -4741010.632 m

Z                3489031.436       3489031.430 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

CC02_GPS

LATITUDE     33 26  5.65768N   33 26  5.64127N

LONGITUDE   117 14 38.37162W  117 14 38.32743W

ELLIP. HT.           299.974           300.729 m

X               -2439193.508      -2439192.909 m

Y               -4737207.078      -4737208.409 m

Z                3494476.753       3494476.748 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

CC03_GPS

LATITUDE     33 25 55.33327N   33 25 55.31686N

LONGITUDE   117 13 38.83761W  117 13 38.79343W

ELLIP. HT.           236.843           237.599 m

X               -2437882.193      -2437881.594 m

Y               -4738019.898      -4738021.229 m

Z                3494176.509       3494176.504 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

CC04_GPS

LATITUDE     33 03 53.80729N   33 03 53.79104N

LONGITUDE   117 17 36.35896W  117 17 36.31485W

ELLIP. HT.            19.077            19.838 m

X               -2453489.002      -2453488.403 m

Y               -4754884.947      -4754886.281 m

Z                3460007.677       3460007.673 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

CC05_GPS

LATITUDE     33 03 55.17472N   33 03 55.15847N

LONGITUDE   117 16 44.53121W  117 16 44.48711W

ELLIP. HT.            62.362            63.124 m

X               -2452300.266      -2452299.667 m

Y               -4755513.093      -4755514.428 m

Z                3460066.597       3460066.594 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

CC06_GPS

LATITUDE     32 59 43.34443N   32 59 43.32812N

LONGITUDE   116 51 29.29360W  116 51 29.24979W

ELLIP. HT.           429.163           429.937 m

X               -2419349.551      -2419348.953 m

Y               -4777446.550      -4777447.887 m

Z                3453762.014       3453762.014 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

CC08_GPS

LATITUDE     33 03  5.15507N   33 03  5.13868N

LONGITUDE   116 34 49.02599W  116 34 48.98235W

ELLIP. HT.          1473.023          1473.803 m

X               -2395028.602      -2395028.005 m

Y               -4786874.941      -4786876.279 m

Z                3459544.446       3459544.448 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

CC09_GPS

LATITUDE     33 01 28.18744N   33 01 28.17107N

LONGITUDE   116 33 12.81931W  116 33 12.77569W

ELLIP. HT.          1474.808          1475.590 m

X               -2393524.532      -2393523.935 m

Y               -4789450.047      -4789451.385 m

Z                3457040.573       3457040.576 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

CC10_GPS

LATITUDE     33 13 22.41283N   33 13 22.39653N

LONGITUDE   117 20 39.64624W  117 20 39.60205W

ELLIP. HT.           -19.942           -19.184 m

X               -2453298.931      -2453298.332 m

Y               -4744166.561      -4744167.893 m

Z                3474653.586       3474653.581 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

CC11_GPS

LATITUDE     33 14 42.82943N   33 14 42.81312N

LONGITUDE   117 19 22.52610W  117 19 22.48192W

ELLIP. HT.           -15.225           -14.468 m

X               -2450903.542      -2450902.943 m

Y               -4743880.680      -4743882.012 m

Z                3476728.418       3476728.413 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

CC12_GPS

LATITUDE     33 24 26.57693N   33 24 26.56054N

LONGITUDE   117 16 27.45360W  117 16 27.40941W

ELLIP. HT.           275.803           276.558 m

X               -2442459.668      -2442459.070 m

Y               -4737392.994      -4737394.325 m

Z                3491915.516       3491915.510 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

CC13_GPS

LATITUDE     33 20 26.98967N   33 20 26.97327N

LONGITUDE   117 06 37.53024W  117 06 37.48617W

ELLIP. HT.           200.682           201.443 m

X               -2430722.499      -2430721.901 m

Y               -4747917.792      -4747919.124 m

Z                3485709.813       3485709.809 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

CC14_GPS

LATITUDE     33 13 25.40420N   33 13 25.38785N

LONGITUDE   117 06 38.26983W  117 06 38.22580W

ELLIP. HT.           404.540           405.303 m

X               -2434065.417      -2434064.819 m

Y               -4754405.469      -4754406.803 m

Z                3474963.257       3474963.254 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

CC15_GPS

LATITUDE     32 43 25.76138N   32 43 25.74521N

LONGITUDE   116 58  3.20236W  116 58  3.15856W

ELLIP. HT.           121.182           121.958 m

X               -2435761.758      -2435761.160 m

Y               -4787150.973      -4787152.312 m

Z                3428296.719       3428296.720 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

CC16_GPS

LATITUDE     32 49 41.40742N   32 49 41.39126N

LONGITUDE   117 09 30.15234W  117 09 30.10839W

ELLIP. HT.            99.287           100.057 m

X               -2448823.544      -2448822.946 m

Y               -4773421.928      -4773423.265 m

Z                3438014.725       3438014.724 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

CC18_GPS

LATITUDE     32 55  9.47961N   32 55  9.46337N

LONGITUDE   116 58 50.86787W  116 58 50.82400W

ELLIP. HT.           254.719           255.491 m

X               -2431587.658      -2431587.060 m

Y               -4776216.218      -4776217.555 m

Z                3446587.585       3446587.585 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

CC19_GPS

LATITUDE     33 05 43.56287N   33 05 43.54648N

LONGITUDE   116 38 46.16091W  116 38 46.11721W

ELLIP. HT.          1080.727          1081.505 m

X               -2399188.279      -2399187.682 m

Y               -4781443.715      -4781445.052 m

Z                3463420.568       3463420.569 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

CC20_GPS

LATITUDE     33 03 55.11909N   33 03 55.10271N

LONGITUDE   116 39  7.96285W  116 39  7.91916W

ELLIP. HT.          1157.974          1158.752 m

X               -2400540.804      -2400540.207 m

Y               -4782877.965      -4782879.302 m

Z                3460662.921       3460662.923 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

HIGH_GPS

LATITUDE     33 01 17.14908N   33 01 17.13277N

LONGITUDE   116 54 41.33763W  116 54 41.29378W

ELLIP. HT.           424.182           424.955 m

X               -2423082.098      -2423081.500 m

Y               -4773784.328      -4773785.664 m

Z                3456182.843       3456182.843 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

HOSK_GPS

LATITUDE     33 04 29.67753N   33 04 29.66115N

LONGITUDE   116 37 32.98381W  116 37 32.94013W

ELLIP. HT.          1215.911          1216.690 m

X               -2398099.526      -2398098.929 m

Y               -4783506.811      -4783508.148 m

Z                3461586.880       3461586.882 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

S301_GPS

LATITUDE     33 22 31.09698N   33 22 31.08067N

LONGITUDE   117 33 54.58990W  117 33 54.54553W

ELLIP. HT.            -5.936            -5.188 m

X               -2467275.488      -2467274.889 m

Y               -4726459.748      -4726461.078 m

Z                3488789.887       3488789.879 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

S302_GPS

LATITUDE     33 21 19.55175N   33 21 19.53537N

LONGITUDE   117 14 56.80794W  117 14 56.76377W

ELLIP. HT.           184.447           185.204 m

X               -2441794.209      -2441793.610 m

Y               -4741216.671      -4741218.002 m

Z                3487053.655       3487053.650 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

S317_GPS

LATITUDE     32 49  6.35134N   32 49  6.33517N

LONGITUDE   117 08 52.23407W  117 08 52.19013W

ELLIP. HT.            90.205            90.975 m

X               -2448209.614      -2448209.015 m

Y               -4774386.127      -4774387.464 m

Z                3437102.269       3437102.268 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

S318_GPS

LATITUDE     32 54 42.60076N   32 54 42.58451N

LONGITUDE   116 55 52.81886W  116 55 52.77503W

ELLIP. HT.           209.071           209.845 m

X               -2427650.342      -2427649.744 m

Y               -4778680.411      -4778681.749 m

Z                3445867.638       3445867.638 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

S436_GPS

LATITUDE     32 50 39.84378N   32 50 39.82753N

LONGITUDE   116 48  7.48049W  116 48  7.43676W

ELLIP. HT.           390.398           391.176 m

X               -2418762.661      -2418762.063 m

Y               -4787904.730      -4787906.068 m

Z                3439685.197       3439685.198 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

SD03_GPS

LATITUDE     33 19 54.33622N   33 19 54.31983N

LONGITUDE   117 09 31.72522W  117 09 31.68112W

ELLIP. HT.            60.096            60.856 m

X               -2434930.095      -2434929.496 m

Y               -4746250.705      -4746252.037 m

Z                3484792.062       3484792.058 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

SD07_GPS

LATITUDE     33 23 15.88137N   33 23 15.86486N

LONGITUDE   116 38 14.04619W  116 38 14.00240W

ELLIP. HT.          1420.794          1421.567 m

X               -2390602.038      -2390601.441 m

Y               -4766182.961      -4766184.295 m

Z                3490727.143       3490727.143 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

SD08_GPS

LATITUDE     33 21 50.51393N   33 21 50.49753N

LONGITUDE   117 08 37.74476W  117 08 37.70066W

ELLIP. HT.           329.508           330.267 m

X               -2432892.874      -2432892.276 m

Y               -4745337.093      -4745338.425 m

Z                3487930.186       3487930.182 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

SD15_GPS

LATITUDE     33 04 27.64026N   33 04 27.62387N

LONGITUDE   116 35 52.57465W  116 35 52.53099W

ELLIP. HT.          1249.128          1249.907 m

X               -2395798.444      -2395797.847 m

Y               -4784729.154      -4784730.491 m

Z                3461552.404       3461552.406 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

SD21_GPS

LATITUDE     32 49 26.15383N   32 49 26.13756N

LONGITUDE   116 37  9.25249W  116 37  9.20889W

ELLIP. HT.          1064.373          1065.156 m

X               -2404276.609      -2404276.011 m

Y               -4797205.844      -4797207.183 m

Z                3438142.985       3438142.988 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

SD33_GPS

LATITUDE     32 40 13.54418N   32 40 13.52801N

LONGITUDE   116 49 42.97429W  116 49 42.93059W

ELLIP. HT.           188.394           189.175 m

X               -2425614.132      -2425613.535 m

Y               -4795948.955      -4795950.295 m

Z                3423349.900       3423349.902 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

T228_GPS

LATITUDE     33 18 26.62418N   33 18 26.60781N

LONGITUDE   117 12 29.72611W  117 12 29.68198W

ELLIP. HT.            26.097            26.856 m

X               -2439690.783      -2439690.184 m

Y               -4745442.533      -4745443.866 m

Z                3482515.305       3482515.300 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

VITA_GPS

LATITUDE     33 21 51.62707N   33 21 51.61067N

LONGITUDE   117 09 44.29659W  117 09 44.25247W

ELLIP. HT.           130.030           130.789 m

X               -2434339.175      -2434338.577 m

Y               -4744386.857      -4744388.189 m

Z                3487849.124       3487849.120 m

TRANSFORMATION FROM ITRF97 (EPOCH = 12-03-1999)

TO NAD_83 (EPOCH = 12-03-1999)

INPUT COORDINATES   OUTPUT COORDINATES

YUNG_GPS

LATITUDE     33 25 48.77827N   33 25 48.76184N

LONGITUDE   117 08 40.79254W  117 08 40.74842W

ELLIP. HT.           318.630           319.389 m

X               -2431115.291      -2431114.693 m

Y               -4741697.351      -4741698.682 m

Z                3494053.022       3494053.017 m