DESIGNATION  - VALPREDO

PID  - NONE, SET NEW MONUMENT.

N 35 04 34.5

# W 119 00 58.6

New monument required.

Additional recovery notes:

Station is located on the Valpredo Rd overcrossing of Interstate 5 approximately 6 miles northwest from the intersection of Hwy 99 and Interstate 5.

To reach from the Hwy 99 and Hwy 166 interchange go west on Hwy 166 for 0.1 miles from the intersection of Hwy 99 and Hwy 166 to a side road right (Mettler Frontage Rd. W.).  Turn right (north) and go 1.3 miles to paved side road left (Valpredo Rd.).  Turn left (west) and go 2.4mi to the I-5 over-crossing (Bridge no 50-292) and the station on the left.  Station is on the east end of the bridge 15.0 feet south of C/L road, 1.0 feet west of the east end of the east abutment, and 1.0 feet south of the south face of curb